CUTTY SARK

Comparison of 2008 and 2005 Keel Survey’s

| Section | 2008 | 2005 | Difference | Explanation |
| --- | --- | --- | --- | --- |
| 1 | 32ft 10 ins | 33ft 3ins | 5ins long | The 2008 survey measured from the keel but if a measurement was taken from the false keel it would have been several inches longer |
| 2 | 34ft 10ins | 34ft 10ins | None |  |
| 3 | 45ft 11ins | 40ft 1ins | 5ft 10ins short | Here we used the frame numbers as a rule of thumb check. The section starts at frame 45 and ends at frame 76. The calculation is 76-45=31x1.5=46ft 6ins. In all probability the 2008 survey, at 7inches less than the rule of thumb figure is correct. |
| 4 | 56ft 5ins | 46ft 9ins |  | See below |
| 5 | 33ft 0ins | 42ft 3ins |  | See below |
| 4 + 5 | 89ft 5ins | 89ft 0ins | 5ins short | At first glance these figures might seem irreconcilable until you add them together. Between them there is just 5 inches difference. It would seem the 2005 survey took their measurement for section five from the forward edge of the cut-out to support the foremast to the end of the keel. In 2008 a measurement from the cut-out to the scarph joint with section five which was 9ft 10ins. If this is deducted from the 42ft 3ins of 2005 it results in a figure of 32ft 5ins, 7 inches less than 2008. From this point to the stem we measured 42ft 10ins. Using the frame number calculation it runs from frame 107 to frame 135. Therefore 135-107=28x1.5=42ft |
| Totals | 203ft 0ins | 197ft 2ins |  | Summary The five inches gained at the beginning was lost at the end so the only outstanding error is section three. Why there is a difference of five feet ten inches is impossible to explain but the frame numbers suggest the 2008 survey is correct. A further check on the overall length of the keel using the frame spacing alone gives a length of 202 feet and 6 inches. |